ECA56 – ADVANCED WIRELESS COMMUNICATION SYSTEMS

ASSIGNMENT – 1

Cellular Network Evolution, Cellular Planning, Spectrum Allocation

| Item | Details |
| --- | --- |
| Course | ECA56 – Advanced Wireless Communication Systems |
| Assignment | Assignment – 1 |
| Topic | Cellular Network Evolution, Cellular Planning, Spectrum Allocation |
| Assessment | Open-ended Engineering Design Report |
| Maximum Marks | 100 |
| Prepared by | Preethiksha L |
| Register No. | 192412452 |
| Department | ECE |
| Date | 28/10/2026 |

# 1. Introduction

The proposed project is a state-wide wireless network intended to provide a common 5G/6G-ready communication platform across six states. The network must serve very different locations, from dense metropolitan areas to villages, highways, industrial zones and healthcare centres. Hence, the design cannot depend on a single frequency band or a single type of cell.

The alternative design presented here uses a coverage-first planning philosophy. A reliable low-frequency layer is established first, followed by a mid-band capacity layer. High-frequency mmWave cells are introduced only at locations where traffic demand is sufficiently high. This approach reduces unnecessary deployment cost while maintaining a path toward future 6G services.

The design also combines network slicing, edge computing, AI-based resource management, mobility optimization, cybersecurity and energy-saving techniques.

# 2. Stage 1 – Engineering Requirements

## 2.1 Regional Requirement Analysis

| State | User/environment profile | Important requirement | Planning approach |
| --- | --- | --- | --- |
| Tamil Nadu | Cities, factories, ports, highways | Industrial reliability + broadband | Mid-band dense layer with low-band coverage and selected hotspots |
| Karnataka | Technology centres, cities, highways | High data traffic + low latency | Mid-band capacity, edge computing and small cells |
| Kerala | Dense settlements, coastal/hilly areas | Coverage + healthcare + resilience | Low-band coverage with strategically placed mid-band cells |
| Maharashtra | Major metropolitan and industrial regions | Very high capacity | Dense RAN, small cells, edge and selective mmWave |
| Gujarat | Industrial corridors, ports and cities | Industrial IoT + logistics | Mid-band industrial coverage and edge processing |
| Uttar Pradesh | Large rural and urban population | Wide coverage + affordable capacity | Low-band macro network with targeted mid-band expansion |

## 2.2 Service Requirements

| Service | Category | Required characteristics | Proposed solution |
| --- | --- | --- | --- |
| Mobile broadband | eMBB | High throughput | Mid-band, MIMO, small cells and mmWave hotspots |
| Autonomous vehicles | URLLC | Low delay and reliable connectivity | Edge UPF, reserved resources and overlapping coverage |
| Remote healthcare | URLLC/eMBB | Reliable video and low delay | Dedicated slice + edge site + redundant backhaul |
| Factory automation | URLLC/mMTC | Reliable control and many devices | Industrial small cells + edge computing |
| Smart meters/sensors | mMTC | Large device population | Wide-area low-band and efficient device scheduling |
| Emergency services | URLLC / public safety | High availability and priority | Dedicated slice and traffic pre-emption |

## 2.3 Main Constraints

Geographical diversity makes uniform cell planning unsuitable.

Low frequencies provide coverage but normally have less bandwidth available for very high capacity.

High-frequency deployment can deliver large capacity but requires dense sites because of shorter practical range and blockage.

Adding cells increases capacity but also increases site, power, backhaul and maintenance requirements.

More aggressive spectrum reuse can increase capacity while increasing interference-management requirements.

Critical applications need better reliability than ordinary best-effort broadband.

Security must be maintained across radio, transport, core, edge and application interfaces.

Energy consumption must be controlled because the network will contain many radio and computing elements.

# 2.4 Problem Formulation

Let the design variables include the number and locations of macro/small cells, frequency assignment, transmit power, antenna configuration, spectrum allocation, edge-node placement and slice resource allocation.

The optimization objective is to minimize total deployment cost, energy use, interference and latency while maximizing coverage, capacity, reliability and spectral efficiency.

Subject to the constraints of available spectrum, site availability, power limits, required coverage, service latency, reliability targets and a blocking probability of not more than 2%.

## 2.5 Performance Measures

| Parameter | Desired engineering objective |
| --- | --- |
| Coverage | High coverage probability in populated and critical corridors |
| Throughput | Adequate capacity during busy hours |
| Latency | Low end-to-end delay for critical services |
| Reliability | High availability for URLLC and public safety |
| Blocking probability | ≤ 2% for the specified traffic model |
| Spectral efficiency | Increase bits/s/Hz through MIMO and beamforming |
| Cell utilization | Avoid sustained overload |
| Energy | Reduce energy per delivered bit |
| Handover | Low handover failure and interruption |

## Design Decision 1

The design prioritizes reliability and coverage first for essential services, followed by capacity, latency, scalability and cost. Broadband traffic is allowed to use flexible resources, while emergency and URLLC traffic receives protected resources.

# 3. Stage 2 – Spectrum and Cellular Planning

## 3.1 Frequency-Band Evaluation

| Band | Strength | Limitation | Preferred deployment |
| --- | --- | --- | --- |
| Sub-1 GHz | Large coverage area and useful penetration | Limited bandwidth for extreme capacity | Rural areas, highways, IoT and emergency coverage |
| 1–6 GHz | Balanced coverage and capacity | Less range than low-band | Main nationwide 5G capacity layer |
| >24 GHz mmWave | Very large bandwidth | Shorter range and blockage sensitivity | Dense hotspots, factories, stadiums and fixed high-rate links |

The proposed spectrum strategy is therefore not a single-band design. Low-band is treated as the reliability and coverage foundation, mid-band as the workhorse capacity layer, and mmWave as an additional hotspot layer.

## 3.2 Cellular Planning Strategy

| Cell | Purpose | Example location |
| --- | --- | --- |
| Macro gNodeB | Wide-area service and mobility | Rural areas, highways, suburbs |
| Micro/small cell | Capacity enhancement | Urban streets and transport hubs |
| Enterprise/industrial small cell | Reliable local connectivity | Factories and industrial parks |
| mmWave small cell | Extreme local capacity | High-density hotspots |
| Edge node | Local processing | Cities, factories and transport corridors |

# 3.3 Frequency Reuse

Frequency reuse permits the same spectrum to be used again in geographically separated cells. In a modern 5G system, reuse can be close to one because advanced antenna techniques and interference coordination are available. However, dense deployments require careful power control, beam selection and scheduling.

The design therefore uses spatial reuse rather than depending only on large frequency separation. This increases spectral efficiency but requires continuous interference monitoring.

## 3.4 Capacity Enhancement

Cell splitting: introduce additional cells in overloaded regions.

Sectoring: divide a cell into directional sectors to increase spatial reuse.

Carrier aggregation: combine available carriers to increase user data rate.

Massive MIMO: serve multiple users using spatial multiplexing.

Beamforming: focus transmitted energy toward the intended user.

Traffic steering: move users between low-band, mid-band and small-cell layers.

## 3.5 Illustrative Cell Calculation

Assume an urban planning zone produces 7.2 Gbps of busy-hour demand and one effective cell provides 600 Mbps under the selected operating conditions.

Minimum cells = 7.2 Gbps / 0.6 Gbps = 12 cells.

If 20% additional capacity is reserved for traffic growth and variation: Planned cells = ceil(12 × 1.20) = 15 cells.

Thus, 15 cells are planned for this illustrative zone. This is a capacity calculation only; the final count must also satisfy coverage and propagation requirements.

## 3.6 Interference Control

| Problem | Control method |
| --- | --- |
| Co-channel interference | Beamforming, coordinated scheduling and power control |
| Adjacent-channel interference | Filtering and suitable channel separation |
| Dense small-cell interference | Adaptive power and interference-aware association |
| Cell-edge interference | Coordinated resource allocation and antenna optimization |
| mmWave blockage | Multiple beams, fallback to lower-frequency coverage |

## State-wise Radio Plan

| State | Primary layer | Additional layer | Special focus |
| --- | --- | --- | --- |
| Tamil Nadu | Mid-band | Low-band + mmWave hotspots | Manufacturing, ports, cities |
| Karnataka | Mid-band | Small cells + edge | Technology centres and highways |
| Kerala | Low-band + mid-band | Selected small cells | Healthcare and resilience |
| Maharashtra | Mid-band | Dense small cells + mmWave | Very high urban traffic |
| Gujarat | Mid-band | Industrial small cells | Ports and industrial automation |
| Uttar Pradesh | Low-band | Mid-band city/corridor layer | Large-area economical coverage |

## Design Decision 2

A mixed radio architecture is recommended. A uniform mmWave network would be expensive and coverage-limited, while a low-band-only network would not provide sufficient capacity. The combination gives the best practical balance.

# 4. Stage 3 – Quantitative Models

## 4.1 Example Traffic Model

| Traffic source | Active units | Assumed rate | Total |
| --- | --- | --- | --- |
| HD video | 8,000 users | 2.5 Mbps | 20 Gbps |
| Autonomous transport | 3,000 vehicles | 0.8 Mbps | 2.4 Gbps |
| Industrial IoT | 120,000 devices | 0.015 Mbps | 1.8 Gbps |

Illustrative aggregate demand = 20 + 2.4 + 1.8 = 24.2 Gbps.

With a 20% planning margin, required capacity = 24.2 × 1.20 = 29.04 Gbps.

The example shows why capacity must be distributed over many cells instead of being supplied by one large cell.

# 4.2 Bandwidth and Spectral Efficiency

The practical relationship is R = Bη. If an effective spectral efficiency of 4.5 bit/s/Hz is assumed, a 500 Mbps cell requires approximately B = 500/4.5 = 111.1 MHz of effective bandwidth. Since real systems have overhead and varying spectral efficiency, actual planning must use measured or simulated throughput rather than the ideal expression alone.

The Shannon expression C = B log₂(1+SNR) is useful as a theoretical upper limit. Actual 5G throughput is reduced by interference, protocol overhead, channel conditions and scheduling.

# 4.3 Propagation and Coverage

Propagation loss increases with distance and is affected by frequency, antenna characteristics and the environment. As frequency increases, practical blockage becomes more important. The proposed network therefore maintains a lower-frequency coverage layer beneath higher-frequency capacity layers.

Urban planning should consider building penetration and street-level blockage, while rural planning should consider terrain, tower height and large inter-site distances.

# 4.4 Latency Allocation

| Network part | Illustrative contribution | Improvement |
| --- | --- | --- |
| Radio access | 5–8 ms | Efficient scheduling and suitable numerology |
| Backhaul/transport | 5–8 ms | Short routes and resilient high-capacity links |
| Core | 3–5 ms | Cloud-native functions and local UPF |
| Edge application | 2–4 ms | Process close to users |
| Total | Approximately 15–25 ms | Application-specific validation required |

The values are illustrative planning targets rather than guaranteed performance. URLLC applications require application-specific testing.

# 4.5 Erlang-B Example

The Erlang-B equation is used to estimate the probability that an offered traffic load is blocked when all channels are occupied:

B(E,N) = (E^N/N!) / [Σ(k=0 to N) E^k/k!].

For an alternative numerical example, assume 750 busy-hour subscribers and 0.025 Erlang per subscriber. Offered traffic E = 750 × 0.025 = 18.75 Erlangs.

Using the recursive Erlang-B calculation, N = 27 channels gives a blocking probability of approximately 1.60%, satisfying the 2% target.

This calculation intentionally treats the channels as ideal traffic channels. Guard bands, duplexing overhead, control resources and other practical spectrum overheads are excluded from this simplified channel-count calculation. The actual supplied/operator traffic values should replace the assumptions.

# 4.6 Reliability and Mobility

Autonomous vehicles require continuous service as they move between cells. Overlapping coverage, optimized neighbor lists, measurement-based handover and rapid target-cell preparation are required. A lower-frequency layer should remain available so that a vehicle does not lose service when a high-frequency beam is blocked.

For emergency services, reliability is increased by redundant backhaul, protected network slices and priority resource allocation.

## 4.7 Design Decision 3

| State | Main expected bottleneck | Intervention |
| --- | --- | --- |
| Tamil Nadu | Industrial and city capacity | Densification and edge computing |
| Karnataka | Peak urban traffic | Small cells and traffic steering |
| Kerala | Coverage/resilience | Low-band expansion and redundant links |
| Maharashtra | Very high capacity | Dense small cells and selective mmWave |
| Gujarat | Industrial reliability | Industrial RAN and edge |
| Uttar Pradesh | Coverage over large area | Low-band macro expansion and targeted mid-band |

# 5. Stage 4 – Architecture Evaluation

## 5.1 Alternative 1: Coverage-Oriented Macro Network

Alternative 1 uses mostly macro cells with low-band and mid-band spectrum. Its major advantage is simpler deployment with fewer sites. It is suitable for broad rural coverage but is less effective for very dense traffic and short-latency enterprise applications.

## 5.2 Alternative 2: Layered Heterogeneous Network

Alternative 2 uses macro cells, small cells, mid-band spectrum, selective mmWave, edge computing and service-aware network slicing. It requires more planning and investment but supports a much wider range of applications.

## 5.3 Decision Matrix

| Criterion | Weight | Alternative 1 | Alternative 2 |
| --- | --- | --- | --- |
| Coverage | 20 | 5 | 4 |
| Capacity | 20 | 3 | 5 |
| Latency | 15 | 3 | 5 |
| Reliability | 15 | 4 | 5 |
| Scalability | 10 | 3 | 5 |
| Complexity | 5 | 5 | 3 |
| Energy | 5 | 4 | 4 |
| Economic efficiency | 10 | 5 | 3 |
| Weighted result | 100 | 3.90/5 | 4.50/5 |

Alternative 2 is selected because the assignment requires support for high-speed broadband, autonomous transport, healthcare, industrial automation, IoT and emergency communication at the same time.

## 5.4 Network Slices

| Slice | Users | Main policy |
| --- | --- | --- |
| eMBB | Broadband/video users | Maximize throughput |
| URLLC | Vehicles, industry, healthcare | Reserve low-latency resources |
| mMTC | Sensors and meters | Support large device numbers efficiently |
| Public safety | Emergency organizations | Highest priority during incidents |

## 5.5 Edge Placement

Edge resources are recommended near major cities, industrial parks, transport corridors and healthcare clusters. Local UPF and application processing can reduce the distance that critical traffic travels before reaching its application.

## Design Decision 4

The heterogeneous design remains the preferred architecture. If the objective changes to rural coverage only, more low-band macro cells and fewer small cells would be selected. If vehicle reliability becomes dominant, more edge nodes, redundant links and overlapping transport-corridor coverage would be added.

# 6. Stage 5 – Integrated Network Design

## 6.1 Proposed Architecture

User equipment and IoT devices connect to gNodeBs through the selected low-band, mid-band or mmWave layer. The RAN connects through redundant backhaul to the 5G Core. AMF provides access and mobility management, SMF manages sessions, and UPF carries user traffic. Local UPF and edge application servers are used for latency-sensitive traffic.

Logical flow:

Users / Vehicles / IoT → gNodeB → 5G RAN → Redundant Backhaul → 5G Core → UPF / Edge → Applications / Cloud

## 6.2 Integrated Control Functions

| Function | Role |
| --- | --- |
| Radio resource management | Assign time-frequency resources according to traffic and radio conditions |
| Mobility management | Maintain service during cell changes |
| Slice management | Reserve and prioritize resources by service |
| AI controller | Predict traffic and recommend optimization actions |
| Security controller | Monitor authentication, anomalies and incidents |
| Energy controller | Reduce unnecessary radio and computing power |

## 6.3 AI-Based Optimization

The AI system receives cell utilization, user counts, signal quality, traffic history and mobility information. It predicts congestion and recommends resource allocation before the cell becomes overloaded.

Predict traffic load for the next planning interval.

Identify cells approaching congestion.

Shift users to available neighboring layers.

Adjust radio resources and transmit power.

Activate or deactivate suitable small-cell capacity.

Compare the result with a conventional rule-based baseline.

## 6.4 Security and Resilience

Strong authentication should be applied to users and network functions.

Control and user-plane traffic should be protected using appropriate encryption.

Anomaly detection should monitor unusual signaling and traffic behaviour.

Critical network functions should have redundancy where economically justified.

Backhaul should use alternate paths so a single link failure does not isolate a critical region.

Emergency traffic should be allowed priority access through the public-safety slice.

Recovery procedures should define how services are restored after equipment or transport failure.

## 6.5 Energy Efficiency

Use low-power operation or sleep modes for lightly loaded small cells when service permits.

Use traffic-aware resource allocation to avoid unnecessary active resources.

Share towers, sites, power and transport infrastructure where possible.

Use efficient cooling and power management for edge sites.

Monitor energy per delivered bit as a long-term KPI.

# 6.6 6G Evolution

The network should reserve an evolution path for AI-native networking and Integrated Sensing and Communication (ISAC). These concepts can be evaluated through research and pilot deployments. Future THz communication may support extremely high data rates at short distances, but it should not be treated as a current nationwide access layer.

## Design Decision 5

The final design intentionally separates current 5G functions from future 6G concepts. 5G NR, gNodeBs, 5G Core, massive MIMO, beamforming, slicing and edge computing form the practical base. AI-native networking, ISAC and future THz systems are treated as evolutionary research directions.

# 7. Stage 6 – Validation

## 7.1 Validation Plan

| Parameter | Validation technique | Result to observe |
| --- | --- | --- |
| Traffic | Analytical model/Python | Peak demand and capacity margin |
| Blocking | Erlang-B | Probability compared with 2% target |
| Coverage | Link budget / propagation model | Coverage probability and dead zones |
| Throughput | MATLAB/ns-3/system model | User and cell throughput |
| Latency | End-to-end model | Application-specific delay |
| Handover | Mobility simulation | Handover success and interruption |
| Interference | Radio simulation | SINR and throughput degradation |
| Energy | Power model | Energy per delivered bit |

## 7.2 Peak Traffic Scenario

If video traffic increases sharply during a busy-hour event, the AI controller identifies overloaded cells and steers users to nearby small cells or alternate carriers. URLLC and public-safety slices remain protected. If overload persists, temporary capacity can be added or additional spectrum activated.

## 7.3 Infrastructure Failure Scenario

If a backhaul link fails, routing should move critical traffic to an alternate path. If an edge node fails, applications should use a redundant edge node or controlled regional-cloud fallback. During an emergency, ordinary broadband traffic may be deprioritized so that public-safety traffic continues.

## 7.4 Verification Table

| Requirement | Design response | Verification |
| --- | --- | --- |
| Wide-area coverage | Low-band macro layer | Coverage analysis |
| High urban capacity | Mid-band + small cells | Throughput/utilization analysis |
| Hotspot capacity | mmWave | Hotspot simulation |
| 2% blocking | Erlang-B sizing | Traffic calculation |
| Low latency | Edge + URLLC slice | Latency analysis |
| Mobility | Overlapping cells + handover | Mobility simulation |
| Massive IoT | mMTC slice | Device-load evaluation |
| Emergency reliability | Priority slice + redundancy | Failure scenario |
| AI optimization | Prediction + dynamic RRM | Baseline comparison |
| Sustainability | Sleep modes + sharing | Energy model |

# 8. Economic, Environmental and Operational Evaluation

| Factor | Positive effect | Trade-off |
| --- | --- | --- |
| Economic | Layered deployment avoids universal mmWave cost | Small-cell and edge deployment increases capital cost |
| Operational | AI and automation reduce manual optimization effort | Requires skilled monitoring and model management |
| Environmental | Energy-aware operation reduces power use | More sites may increase total equipment footprint |
| Public safety | Dedicated resources improve emergency availability | Redundant infrastructure increases cost |
| Scalability | Cloud-native core and slices support new services | More software orchestration is required |

# 9. Limitations and Further Data Required

The question paper does not provide numerical subscriber counts, busy-hour traffic per subscriber or actual channel counts; therefore, the numerical examples are stated assumptions.

Actual spectrum availability and licensing conditions must be confirmed before implementation.

Site surveys are required to determine tower locations, antenna heights, terrain and building losses.

Measured traffic traces are required to replace the illustrative traffic model.

Real propagation and system simulations should be used before final cell-count decisions.

Security and regulatory compliance must be verified against the applicable operational environment.

# 10. Final Recommendation

The recommended solution is a phased heterogeneous 5G/6G-ready network. The first layer should provide reliable low-band coverage across rural regions, highways and critical public-safety areas. The second layer should provide the majority of capacity through mid-band macro and small cells. The third layer should use mmWave only at locations where traffic demand justifies it.

The network should use a 5G Core with network slicing and local edge processing. Four logical slices should support eMBB, URLLC, mMTC and public safety. AI-based resource management should be introduced after collecting adequate network measurements, and its performance should be compared with a baseline method.

The recommended deployment is therefore 'deployment with modifications and validation'. The design is technically promising, but actual implementation requires operator-specific spectrum, traffic, propagation, site and cost data.

# 11. Conclusion

A six-state communication system cannot be optimized effectively with one spectrum band or one cell type. The alternative design uses a layered approach so that each frequency range performs the role for which it is most suitable. Low-band provides wide coverage, mid-band supplies the main capacity, and mmWave adds localized high-throughput capability.

The combination of OFDMA, massive MIMO, beamforming, frequency reuse, small-cell densification, network slicing and edge computing supports the diverse requirements of broadband, autonomous transport, healthcare, industrial automation, IoT and emergency services. AI-based optimization can further improve traffic management and energy efficiency.

The quantitative models demonstrate how traffic demand, spectral efficiency, cell sizing, latency and Erlang-B blocking can support engineering decisions. Final deployment should replace all illustrative values with measured network data and should be validated using MATLAB, Python, ns-3 or equivalent tools.